# Annex II. Description of the codes used

COBRA-I-IV

The COBRA-I-IV (COolant Boiling in Rods Array) is a core subchannel code belonging to the COBRA code family including COBRA-IIIC, VIPRE, COBRA-TF [1]. It is able to compute the thermalhydraulic properties in nuclear fuel rod bundles or cores in steady-state and transient conditions.

The code can operate either an explicit or implicit solution scheme to solve the balance equations of mass, momentum and energy using a finite difference scheme applied to Eulerian computational control volumes. The control volumes can range as little as the fuel assembly subchannels up to core fuel assemblies in radial, azimuthal and axial direction.

The implicit solution scheme was inherited by the COBRA-IIIC version, where the equations of mass, momentum and energy are solved using a lumped finite difference scheme. But COBRA-I-IV also provides an explicit solution scheme that allows the calculation of transients where there are experienced phenomena like flow reversals, re-circulations, and expulsion and re-entry flows, with either an inlet flow or pressure drop boundary condition specified.

The fuel thermal response is also considered and chained to the thermalhydraulic fluid response by use of Ushakov heat transfer constitutive correlation [2]:

Nu=7.55pd-20pd13+0.041pd2Re Pr0.56+0.19pd				[1]

Where,

pd, pitch-to-diameter ratio,

Re, Reynolds number

Pr, Prandt number

Nu, Nusselt number

The friction factor, used to calculate the pressure drop in the sub-channel, is derived from Rehme correlation [3].

Additional model is applied to simulate the behaviour of wire wraps [4]. The model is based on assumption that the crossflow that is induced by the wire-wrap passing through a gap is a fraction of the subchannel flow. The cross-vs- axial split is computed as:

wforced=π(D+t)(sAi)(δDx)mi					[2]

Where,

D, rod diameter,

t, wire wrap diameter

s, gap width,

Ai, area of subchannel flow rate,

Dx, axial node length,

Δ, arbitrary parameter, usually set to Dx/wire-wrap pitch length.

When wire-wrap model is called, subchannel flow areas and wetted perimeters are adjusted to take account for the presence of a wire-wrap in the subchannel. The forced crossflows are then kept constant while solving the transverse momentum equations for the remaining crossflows.

COBRA is also able to compute axial fluid and fuel conduction that is particularly relevant for liquid metal studies.

## References for COBRA-I-IV

1. Wheeler, C. L., Stewart, C. W., Cena, R. J., et al., "COBRA-IV-I: an interim version of COBRA for thermal-hydraulic analysis of rod bundle nuclear fuel elements and cores," (1976), https://doi.org/10.2172/7359476

2. Mikityuk K. “Heat transfer to liquid metal: Review of data and correlations for tube bundles”. Nuclear Engineering and Design 239 (2009) 680–687

3. Rehme, K., 1973. Pressure drop correlations for fuel element spacers. Nuclear Technology

4. Donovan, T E, George, T L, and Wheeler, C L, "COBRA-IV wire wrap data comparisons. [LMFBR]," (1979), https://doi.org/10.2172/6043692