# Description of simulation codes(FRTAC,CIAE)

Based on the modular concept, the China Institute of Atomic Energy has independently developed a safety analysis software FRTAC(Fast Reactor Transient Analysis Code)using the C++ programming language for simulating the normal operation, expected operational events, design basis accidents, and super design basis accidents of liquid metal cooled reactors.

In 2012, when FRTAC was initially developed, it employed a display algorithm and was a system analysis program solely for sodium-cooled fast reactors. After 2018, the version changed to use a semi-implicit algorithm. In 2019, multi-threaded parallel computing functionality was introduced. After 2020, the program gradually added properties such as lithium, lead/lead-bismuth, and carbon dioxide, further expanding the program's application scope and gradually evolving into a general system transient analysis program for liquid metal-cooled reactors.

In 2012,during the initial development of FRTAC,an explicit algorithm was adopted. It was a system analysis code designed solely for sodium-cooled fast reactors.The version after 2018 has adopted the semi-implicit algorithm. In 2019, multi-threaded parallel computing capabilities were introduced. After 2020, the code progressively added physical properties for lithium, lead/lead-bismuth, carbon dioxide, and other materials, further expanding its application scope. It gradually evolved into a universal system transient analysis program for liquid metal cooled fast reactor. In 2022, developers optimized the program's algorithm and added a visual interface, significantly enhancing its user-friendliness.

Currently, the FRTAC development covers multiple majors such as reactor neutron dynamics, reactor thermal-hydraulics, reactor safety, reactor systems , reactor control, etc. It has developed several calculation modules including thermal-hydraulic solution, heat transfer solution for heat structures, neutron dynamics solution, water-vapor conversion, blowdown, etc. It includes various control volume such as hydraulic components, heat structures, neutron components, etc. in the reactor, and involves multiple flow media such as water/vapor, sodium, lead/lead-bismuth, air, helium, carbon dioxide, etc. The entire process of modeling, solution and post-processing has a visual interface.The code can conduct steady-state and transient process simulation analysis for thermal systems with liquid lead/bismuth, liquid sodium, water and other fluids as the medium. It can also model and simulate the thermal-hydraulic responses during the transient and steady-state operation of fast reactor systems, as well as various accident processes such as super-power, loss of coolant accident, and loss of heat sink in fast reactors.