- HYDRA-IBRAE/LM

System thermal-hydraulic code HYDRA-IBRAE/LM [1] is being developed in the Nuclear Safety Institute of the Russian Academy of Sciences (IBRAE RAN) in frame of Proryv Project for safety justification of reactor facilities with lead, lead-bismuth, and sodium coolants. The code also allows to simulate water and air circuits of these facilities. The simulation of thermal-hydraulic processes in the code is based on a non-equilibrium two-fluid model in a one-dimensional approximation taking into account admixture of noncondensable gases. A three-fluid model can be used to simulate the dispersed-annular flow regime of water.

There are a number of special models in the code, for example: heat transfer in solids in 1D and 2D; radiative heat transfer; lead coolant freezing/melting processes. There is also the possibility of individual equipment elements simulation: pumps, valves, and turbines. To perform uncertainty and sensitivity analysis, a specialized utility developed in IBRAE RAN as well, can be used. It allows running multivariate calculations with variation of input parameters and performing statistical processing of the obtained results. HYDRA-IBRAE/LM is a module of the integral codes EUCLID/V1 and EUCLID/V2. The EUCLID/V1 code is used for multiphysical simulation of reactor facilities in normal operation and abnormal operation, including the initial stages of accidents up to the moment of loss on integrity of fuel rod cladding and release of radioactive substances. The EUCLID/V2 code can be also used for severe accidents simulation.

The code has an extensive verification and validation base. For validation, experimental data obtained at sodium-cooled reactors (BN-600, BN-800, and BOR-60) was used. More than 30 experimental facilities with sodium, heavy liquid metal [2], water, and air were simulated. The code is also used to simulate the natural circulation of liquid metal coolant [3].

## REFERENCES

1. Alipchenkov V.M., Anfimov А.М., Afremov D.А., Gorbunov V.S., Zeigarnik Yu.А., Kudryavtsev А.V., Osipov S.L., Mosunova N.A., Strizhov V.F., Usov E.V., Fundamentals, Current State of the Development of, and Prospects for Further Improvement of the New-Generation Thermal-Hydraulic Computational HYDRA-IBRAE/LM Code for Simulation of Fast Reactor Systems, Thermal Engineering. 2, 130-139 (2016) https://doi.org/10.1134/S0040601516020014

2. Alipchenkov V.M., Grudtsyn Ya.V., Ilyasova O.H., Mosunova N.A., Simulation of the processes occurring during steam generator tube rupture in the lead cooled reactor unit using HYDRA-IBRAE/LM code, Progress in Nuclear Energy, V. 149, 104278. (2022) https://doi.org/10.1016/j.pnucene.2022.104278

3. Ilyasova O.H., Alipchenkov V.M., Mosunova N.A., Grudtsyn Ya.V., Simulation of natural circulation experiments for heavy liquid metal coolant using HYDRA-IBRAE/LM code. Progress in Nuclear Energy, V. 173, 105233 (2024)DOI: https://doi.org/10.1016/j.pnucene.2024.105233

- CELSIST

The sub-channel code CELSIST is designed for the thermal-hydraulic calculation of fuel assemblies with a single phase liquid metal coolant. The CELSIST sub-channel module solves a system of finite-difference conservation equations of mass, momentum and energy. The thermal conductivity equations are being solved in the sectors of fuel rods in accordance with the wetted perimeters in the cells.