# NekRs and Cardinal

The NekRS [1] code is a next-generation highly scalable CFD code developed by the Exascale Computing Project (ECP) and the Nuclear Energy Advanced Modeling and Simulation (NEAMS) programs. It uses the spectral element method, which is a subset of the finite element method. It uses high-order Lagrange polynomials as the basis functions for spatial approximation with Gauss-Lobatto-Legendre (GLL) quadrature. This has the advantage of exponential convergence in spatial resolution error with increasing order. Spectral element methods also have negligible numerical diffusion, which is a distinct advantage for high-fidelity methods. It has demonstrated scalability on the current leadership class computers at the Oak Ridge and Argonne Leadership Computing Facilities [2] as well as impressive scalability to enable full-core reactor geometries [3].

Cardinal [4] is a NEAMS multiphysics tool which provides the capability to couple high-fidelity CFD (using NekRS) to high-fidelity neutronics (using OpenMC) and the MOOSE environment. By coupling to the MOOSE environment, Cardinal allows NekRS to be coupled with all the capabilities offered by the MOOSE framework. In this work, the solid heat conduction module in MOOSE was used for a conjugate heat transfer (CHT) capability. The advantage of this approach is that it allows the solid domain mesh to be non-conformal to the fluid domain mesh.

# References for ANNEX II

- P. Fischer, S. Kerkemeier, M. Min, et al., “NekRS, a GPU-Accelerated Spectral Element Navier-Stokes Solver”, Parallel Computing. 114, pp 102982 (2022) doi.org/10.1016/j.parco.2022.102982

- M. Min, Y.-H. Lan, P. Fischer, et al., “Nek5000/RS Performance on Advanced GPU Architectures”, ANL-22/81, Argonne National Laboratory, 2022 doi.org/10.2172/1894022

- D. Shaver, J. Fang, H. Yuan, et al., “Full-Core Computational Fluid Dynamics Simulations Using NekRS”, Proc. of the 20th Int’l Topical Meeting on Nucl. Reactor Thermal Hydraulics (NURETH-20). Washington DC 2023

- A. Novak, D. Andrs, P. Shriwise, et al., “Coupled Monte Carlo and Thermal-Fluid Modeling of High Temperature Gas Reactors Using Cardinal”, Annals of Nucl. Energy. 177, pp 109310 (2022) doi.org/10.1016/j.anucene.2022.109310