- BARAM CFD with OpenFoam

OpenFoam (Open Field Operation and Manipulation) ([1]) is a free, open-source computational fluid dynamics (CFD) software package written in C++ and licensed under GNU General Public License. It is a C++ toolbox designed for developing customized numerical solvers and pre-/post—processing utilities for solving continuum mechanics problems, particularly in fluid dynamics, heat transfer, thermodynamics, and related phenomena such as chemical reactions, turbulence and fluid-structure interactions. OpenFoam provides a vast library of pre-built solvers and utilities, enabling users to model complex physical systems through a flexible, customizable framework. chtMultiRegionFoam solver is a steady-state or transient pressure-based CFD solver designed for simulating conjugate heat transfer (CHT) between multiple fluid and solid regions, including buoyant, turbulent flow and solid heat conduction. It is capable of handling arbitrary fluid and solid regions, supports compressible flow, and incorporates model for turbulence (RAS, LES, DES), radiation and heat souces. It employs a segregated solution strategy, where fluid and solid equations are solved sequentially, with coupling achieved iteratively using temperature from  the previous iteration to define boundary conditions at the interfaces until convergence is reached.

BARAM CFD ([2]), currently v25,  is a free, open-source CFD software package designed to make CFD analysis more accessible by providing an intuitive GUI over the OpenFoam engine. Developed by NEXTfoam Inc., BARAM aims to reduce the steep learning curve typically associated with text-based CFD solvers, enabling users to focus on engineering problems rather than complex command-line setups. It supports both Linux and Windows , the setup providing on chosen operating system the OpenFoam engine, so being a fully cross-platform GUI-based OpenFoam distribution. There are two separate graphical interfaces:

- BARAM Mesh, the mesh generation module of BARAM, which utilizes OpenFoam utilities, blockMesh and snappyHexMessh, to generate computational meshes for CFD simulations. The module allows users to import geometry from STL files or create simple primitive geometries such as hexahedrons, spheres and cylinders. It can generate 3D multiregion meshes including boundary layers, supporting the creation of regions, cell zones, interfaces and baffles.

- BARAM Flow, the simulation module within BARAM software, designed for analyzing incompressible and compressible flows, heat transfer, multiphase flow and chemical species mixing. It organizes projects into folders along with key files such as logs and baram.foam, the later being specifically used for post-processing in ParaView ([3]), an open-source visualization tool.

References:

[1] https://www.openfoam.com/news/main-news/openfoam-v2412

[2] https://baramcfd.org/

[3] https://www.paraview.org/