PUCHOK-GMT subchannel code description (section 3.9 of Appendix 2)

The Puchok-GMT code is a program written in Fortran and designed for the thermal-hydraulic analysis of steady-state cooling regimes in fuel assemblies (FAs) with a single-phase liquid metal coolant. Fuel rod spacing can be achieved either via spacer grids or via wire wrapping for two configurations: spacer rib-on-cladding and rib-on-rib.

The code provides the spatial distribution of cell-averaged coolant temperatures and fuel rod temperatures (cladding, gap, and fuel core), taking into account hot channel heat flux factors (over-heating factors, which ensures conservative results).

The mathematical model for coolant flow in the FA is based on the steady-state conservation equations for mass, momentum, and energy, written for average quantities in elementary cells. The physical flow model is characterized by the following features:

- the hydraulic and thermal non-equivalence of cells is accounted for by specifying fuel rod parameters in each elementary cell.

- the flow distribution among cells is achieved by equalizing longitudinal pressure gradients in the cells at each iteration step using an iterative method.

- several empirical correlations for the turbulent exchange coefficient are implemented in the code to describe turbulent exchange.

- integration of the thermal-hydraulic equations for the system of interconnected elementary channels is performed using the Euler method with recalculation and automatic step selection depending on the specified error level;

- hydraulic friction losses in cells are determined considering empirical corrections for the geometric shape of the elementary cells.

- the methodology for refining closure relations for hydrodynamic processes is based on the application of numerical solutions obtained with the 3D CFD code STAR-CCM+. The friction drag coefficients and Nusselt numbers of regular and irregular cells (shape factors relative to a regular cell) are found from the three-dimensional solution by averaging.

For each fuel rod, a two-dimensional steady-state heat conduction problem is solved in cross-sections by establishing a non-stationary problem. The code accounts for the influence of wire wrapping, an oxide film, and the non-uniform axial power distribution in the FA on the heat transfer coefficient. Thermophysical properties of the liquid metal coolant are specified as temperature-dependent polynomials.

The code accounts for the following transverse mixing mechanisms: convective flow redistribution, turbulent exchange across cell boundaries, cross-flows at spacer grids, cross-flows in peripheral cells due to flow swirling, heat transfer within fuel rods, and heat and mass exchange at the boundaries of a given FA with adjacent FAs, as well as with the environment (in the case of a FA with a duct). For a ducted FA, the conditions outside the duct can be specified either in a tabular form (heat transfer coefficient, ambient temperature as a function of the axial coordinate along the FA) or by modeling a system of hydraulically independent inter-assembly gaps with specified coolant flow rates.

The code has a modular structure. The following are implemented as separate subroutines: the Euler integration method block, calculation of axial derivatives for pressure, flow rates and enthalpies, calculation of thermophysical properties, solution of main balance equations for spacer grids, calculation of inter-channel mixing coefficients, and solution of the heat conduction problem in a fuel rod.

Limitations on the code's area of application are as follows: the number of cells must not exceed 500, the number of connections between cells must not exceed 700, and the number of fuel rods must not exceed 500. These limitations are imposed due to the stability of the numerical method when calculating cross-flows between cells. The implemented closure correlations are validated for a triangular (hexagonal) fuel rod lattice.