# RELAP5

RELAP5 series of codes has been developed at the Idaho National Laboratory (INL) under sponsorship of the U.S. Department of Energy (DoE), the U.S. Nuclear Regulatory Commission (NRC), members of the International Code Assessment and Applications Program (ICAP), members of the Code Applications and Maintenance Program (CAMP), and members of the International RELAP5 Users Group (IRUG). Specific code applications have included simulations of transients in Light Water Reactors (LWR) such as loss of coolant, anticipated transients without scram, and operational transients such as loss of feedwater, loss of offsite power, station blackout, and turbine trip. RELAP5 is a highly generic code that, in addition to calculating the behavior of a reactor coolant system during a transient, can be used for simulation of a wide variety of hydraulic and thermal transients in both nuclear and nonnuclear systems involving mixtures of vapor, liquid, noncondensable gases, and nonvolatile solute.

The mission of the RELAP5 development program was to develop a code suitable for the analysis of all transients and postulated accidents in LWRs, including large and small-break loss of coolant accidents as well as the full range of operational transient applications. However, the code can also be used for space reactor simulations, gas cooled reactor applications, fast breeder reactor modelling, and nuclear fusion reactor calculations.

The RELAP5 code is based on a nonhomogeneous and nonequilibrium model for the two-phase system that is solved by a fast, partially implicit numerical scheme to permit economical calculation of system transients. The RELAP5 development effort was aimed at producing a code that included important first-order effects necessary for accurate prediction of system transients but that was sufficiently simple and cost effective so that parametric or sensitivity studies can be carried out.

The code includes many generic component models from which general systems can be simulated, including pumps, valves, pipes, heat releasing or absorbing structures, reactor kinetics, electric heaters, jet pumps, turbines, compressors, separators, annuli, pressurizers, feedwater heaters, ECC mixers, accumulators, and control system components. In addition, special process models are included for effects such as form loss, flow at an abrupt area change, branching, choked flow, boron tracking, and noncondensable gas transport.

The hydrodynamic model and the associated numerical scheme are based on the use of fluid control volumes and junctions to represent the spatial character of the flow. The control volumes can be viewed as stream tubes having inlet and outlet junctions. The control volume has a direction associated with it that is positive from the inlet to the outlet. Velocities are located at the junctions and are associated with mass and energy flow between control volumes. Control volumes are connected in series, using junctions to represent a flow path. All internal flow paths, such as recirculation flows, must be explicitly modelled in this way since only single liquid and vapor velocities are represented at a junction. (In other words, a counter-current liquid-liquid flow cannot be represented by a single-junction.)

Heat flow paths are also modelled in a one-dimensional sense, using a finite difference mesh to calculate temperatures and heat flux vectors. The heat conductors can be connected to hydrodynamic volumes to simulate a heat flow path normal to the fluid flow path. The heat conductor or heat structure is thermally connected to the hydrodynamic volume through a heat flux that is calculated using heat transfer correlations. Electrical or nuclear heating of the heat structure can also be modelled as either a surface heat flux or as a volumetric heat source. The heat structures are used to simulate pipe walls, heater elements, nuclear fuel pins, and heat exchanger surfaces.

In the last decades, the code has been extensively used for the simulation of liquid metal fast reactors, as discussed in many papers available in the most important journals of the nuclear field. A comprehensive review is available in [1].

Referring to this benchmark activity, different versions of the code are currently used, such as RELAP5-3D [2], RELAP5/Mod3.3 [3] and RELAP/SCDAPSIM/MOD 4.1 [4].

- P. Lorusso et al., State-of-the-art review of the T/H system codes RELAP5 for HLM applications, Nuclear Engineering and Design, vol. 414, Article n° 112627 (2023). https://doi.org/10.1016/j.nucengdes.2023.112627.

- The US Nuclear Regulatory Commission (USNRC), RELAP5-3D© Code Manual: Structure, System Models, and Solution Methods, INL/MIS-15-36723, Revision 4.3, Volume I, Idaho Falls, Idaho (USA) 2015

- The US Nuclear Regulatory Commission (USNRC), RELAP5/MOD3 Code Manual: Code Structure, System Models, and Solution Methods, NUREG/CR-5535, USNRC, Volume I, Washington DC (USA) 1998.

- C. M. Allison et al., The development of RELAP5/SCDAPSIM/MOD4.0 for reactor system analysis and simulation, in Proceedings of the 7th International Conference on Nuclear Option in Countries with Small and Medium Electricity Grids, Dubrovnik, Croatia, May 2008.