- RELAP5

RELAP5/SCDAP 4.1.0 BUILD 12/2018 was obtained as RELAP/SCDAPSIM/MOD4.1 from Innovative System Software in 2020, in the frame of a four years user agreement, as binary version on Windows. MOD4 ([1]) uses PbBi eutectic physical properties from [2]. The RELAP5/MOD3 code, on which relies RELAP/SCDAPSIM, is a 1D thermal-hydraulic analysis code that has been developed originally for best-estimate transient simulation of light water reactor coolant systems during postulated accidents. It includes hydrodynamic components such as pipe, volume, etc., and heat structures that can describe solid material regions, heat being transferred from/to volumes, or by means of boundary conditions (BCs). Addition of several advanced/innovative reactor coolants has been made, including Pbi eutectic and molten lead. However, capabilities like tracking noncondensible gases are working for water only, and were not extended to advanced coolants.

References:

[1] STDP/ADTP - RELAP/SCDAPSIM User Reference Manual -Advanced Fluid Systems Thermal Hydraulics Analysis, September 2, 2020.

[2] NEA/OECD -Handbook on Lead-bismuth Eutectic Alloy and lead Properties, Materials Compatibility, Thermal-hydraulics and Technologies, 2007 edition.