# SPECTRA

SPECTRA [1] is a thermal-hydraulic system code developed by NRG PALLAS. SPECTRA is designed for thermal-hydraulic analyses of nuclear power plants. The code’s main applicability is in the areas of Light Water Reactors, High-Temperature Reactors, Liquid Metal Reactors, Molten Salt Reactors, as well as conventional plants and chemical reactors. The code can be used for analysing accident scenarios, including loss-of-coolant accidents, operational transients, and other accident scenarios in nuclear power plants. The available models include multi-dimensional two-phase flow, non-equilibrium thermodynamics, transient heat conduction in solid structures, a general heat and mass transfer package, with natural and forced convection, as well as condensation and boiling.

A modelling approach used by the SPECTRA code is based on a series of 1-D control volumes, junctions and solid heat conductors. A control volume (CV) represents a physical room or a compartment characterised by horizontal cross-sections and heights. Each CV is divided into two fields, referred to as a pool and atmosphere. Each field has its temperature and velocity. The thermal-hydraulic (TH) behaviour in a CV is governed by the equations of conservation of mass and energy.

A Junction is a connection between two CVs, through which fluids may be transported from one CV to another. Junctions are characterised by their flow area, flow direction, height (which is related to the flow direction), as well as momentum length, friction length, etc. The momentum balance is solved in the junctions.

A 1-D Solid Heat Conductor is a structure that is represented by one-dimensional heat conduction, with specified boundary conditions at each of its two boundary surfaces. The modelling capabilities of solid heat conductors are general and can represent walls, containment structures, fuel rods with nuclear or electrical heating, piping walls, heat exchangers with smooth or finned tubes, etc. Heat conduction within solid structures and energy transfer across their boundary surfaces into control volumes are solved in the heat conductors. The code structure  is based on Packages, which contain models on a given topic. Detailed information about SPECTRA is reported in the manuals, which are available publicly online [1].

[1] M. Stempniewicz, “SPECTRA - Sophisticated Plant Evaluation Code for Thermal-hydraulic Response Assessment Vol. 1,2,3, Version 25-09,” K6223/25.319650 MSt-2509, September 2025, https://marekstempniewicz.com/spectra.html.