# STAR-CCM+

STAR-CCM+ is a commercial Computational Fluid Dynamics (CFD) software from Siemens Digital Industries Software (USA). It is widely used for the nuclear energy engineering applications to simulate complex fluid flow and heat transfer phenomena. Its integrated environment supports the entire simulation workflow, from geometry creation and meshing to solving and post-processing, within a single user interface. STAR-CCM+ also supports multi-physics simulations involving fluid flow, solid mechanics, multiphase flow, combustion, and electromagnetics.

STAR-CCM+ employs a finite volume method (FVM) to discretize and solve the governing equations of fluid dynamics, including the Navier–Stokes equations for momentum, energy, and mass conservation. It features both segregated and coupled solvers. Segregated solvers solve each equation sequentially, which is more memory-efficient and suitable for incompressible, low-speed flows. Coupled solver solves the momentum and pressure equations simultaneously offering better stability and convergence making it more suitable for compressible or strongly coupled flows. The software supports both steady-state and transient simulations, offering both implicit and explicit time integration schemes for the transient simulations.

The software features a generalized polyhedral cell formulation, enabling robust simulations across various mesh types. The automated meshing tools in the software can produce polyhedral, trimmed, and overset meshes, with support for mesh morphing and adaptive refinement.

STAR-CCM+ offers a comprehensive suite of turbulence modelling approaches to simulate a wide range of flow regimes. The software offers both Reynolds-Averaged Navier–Stokes (RANS) models and higher-fidelity modelling approaches such as Large Eddy Simulation (LES) and Detached Eddy Simulation (DES).  RANS models offered by the software can be classified mainly in two types, namely, the eddy-viscosity models (including k-ε, k-ω and Spalart-Allmaras), and Reynolds stress transport models. Wall treatment options include high-Reynolds number wall functions, low-Reynolds number models, and hybrid near-wall approaches, allowing flexibility based on mesh resolution and flow characteristics.

[1] Siemens, “Simcenter STAR-CCM+ Documentation v2406,” Siemens Digital Industries Software, 2024.