IAEA TECDOC on NACIE CRP

- Introduction (IAEA)

  - Background of the CRP

The Coordinated Research Project (CRP) focused on thermal-hydraulic analysis of the tests performed at the forced to natural circulation experimental (NACIE) facility at ENEA (Italy) in 2017. NACIE-UP is a rectangular loop in which there is a difference in heigth (about 5 m) between heat source (FPS) and heat sink (HX): this allows Natural circulation. Forced circulation is guaranteed by a gas-injection (Argon) in the riser of the facility: the gas will lighten the riser and the hydrostatic unbalance between the two columns will provide a positive pressure head. This method is called gas-lift. Transient analysis of thermal hydraulic phenomena in NACIE loop tests provided an excellent opportunity for validation of the physical and mathematical models and numerical simulation codes using actual experimental data. The experimental data were available both for integral values and local temperature distribution that makes it possible the validation and comparison of reactor system codes, subchannel analysis and CFD simulations versus experimental data.

This publication provides a detailed description of the benchmarking exercise along with the results of the simulations performed by the participants.

  - Objectives of the CRP

The overall objective of the CRP was to improve the Member States' analytical capabilities in the field of simulation of fast reactors cooled by heavy liquid metals.

A necessary condition towards achieving this objective was a wide international validation and qualification effort of the analysis methodology and codes currently employed in the fields of fast reactor neutronics, thermal hydraulics and plant dynamics to achieve enhanced safety. The main aim was to enable Member States to make informed decisions on the development of new or advanced fast reactor designs, and to increase cooperation between Member States in achieving advances in fast reactor technology development through international collaborative R&D.

The CRP was implemented as a programmatic activity of the IAEA Project 1.1.5.3 “Advanced technology for fast reactors” starting with the IAEA Program and Budget Cycle 2022 – 2023. The Project 1.1.5.3 had the objective, among others, to enable Member States to make informed decisions on the development of new or advanced fast reactor designs, and to increase cooperation between Member States in achieving advances in fast reactor technology development through international collaborative R&D. Given its aforementioned overall objective, the CRP clearly responded to the objectives of the IAEA Project 1.1.5.3.

  - Scope of the CRP

The main scope of the CRP was to contribute to validation of the simulation codes and models, investigate the applicability of existing codes to the heavy liquid metals coolants, investigate the heat transfer of the wire wrapped fuel assembly.

  - Structure of the Document

This publication is organized in 8 major Sections including this introduction Section. Section 2 presents the CRP phases and timeline of their implementation. Section 3 gives the overview of the NACIE Facility, and the test selected for this CRP. Section 4 describes the technical specification used for the benchmarking exercise. Section 5 describes the mathematical model: assumptions and simplifications. Section 6 describes the comparison of simulation results by all participants versus experimental observations. Section 7 describes the sensitivity and uncertainty analysis. The main conclusions and recommendations are presented in Section 8.